# ECG Classification for LBBB and Normal Beats

## Introduction

Left Bundle Branch Block (LBBB) is a cardiac conduction abnormality that manifests as prolonged QRS complexes on an Electrocardiogram (ECG). Detecting and classifying LBBB from normal beats involves preprocessing ECG signals, extracting meaningful features, and using machine learning models to categorize them.

This guide outlines a complete pipeline for LBBB detection, comprising the following steps:

- Preprocessing the ECG signal.

- Extracting features using wavelet analysis.

- Classifying beats with machine learning.

## 1. Preprocessing Functions

### 1.1 butter_bandpass_filter(data, lowcut, highcut, Fs, order=5)

- Purpose: Applies a Butterworth bandpass filter to isolate ECG signal components within the desired frequency range.

- Parameters:

  - data: Input ECG signal array.

  - lowcut: Lower cutoff frequency (e.g., 0.5 Hz).

  - highcut: Upper cutoff frequency (e.g., 40 Hz).

  - Fs: Sampling frequency (e.g., 360 Hz).

  - order: Filter complexity (default: 5).

- Why?: This step eliminates baseline drift and high-frequency noise, ensuring the signal emphasizes the relevant P, QRS, and T waves.

### 1.2 remove_baseline_wander(ecg_signal, sampling_rate=1000, cutoff=0.5)

- Purpose: Removes slow baseline drift using a high-pass Butterworth filter.

- Parameters:

  - ecg_signal: Input ECG signal.

  - sampling_rate: Sampling frequency (default: 1000 Hz).

  - cutoff: High-pass filter cutoff frequency (default: 0.5 Hz).

- Why?: Baseline drift interferes with QRS detection and feature extraction. Centering the signal around zero improves accuracy.

### 1.3 reduce_noise(signal, window_length=11, polyorder=3)

- Purpose: Reduces high-frequency noise using a Savitzky-Golay filter.

- Parameters:

  - signal: Input ECG signal.

  - window_length: Size of the filtering window (odd value, e.g., 11).

  - polyorder: Polynomial order (default: 3).

- Why?: Reducing noise preserves the morphology of the QRS complex, improving feature extraction.

### 1.4 detect_qrs_complexes(signal, distance, height)

- Purpose: Detects QRS complexes using peak detection algorithms.

- Parameters:

  - distance: Minimum distance between consecutive peaks (in samples).

  - height: Minimum height for peaks to qualify as QRS complexes.

- Why?: QRS complexes are essential for beat alignment and provide critical features for detecting abnormalities like LBBB.

## 2. Feature Extraction

### 2.1 extract_wavelet_features(signal, wavelet='db3', level=5)

- Purpose: Decomposes the ECG signal into frequency-domain components using the discrete wavelet transform (DWT).

- Parameters:

  - signal: Input ECG signal.

  - wavelet: Type of wavelet used (e.g., 'db3', a Daubechies wavelet).

  - level: Number of decomposition levels (e.g., 5).

- Why?: Wavelets capture transient features in ECG signals, such as variations in the QRS complex, making them ideal for analyzing LBBB.

Note: Daubechies wavelets, such as 'db3', are particularly suited for biomedical signal processing due to their compact support and similarity to ECG morphology.

## 3. Data Loading and Cleaning

### 3.1 clean_data(data)

- Purpose: Ensures data integrity by removing invalid or extreme values.

- Parameters:

  - data: Input signal array.

- Why?: Cleaning avoids computational errors and improves preprocessing accuracy.

### 3.2 load_data(filename, label, Fs=1000)

- Purpose: Loads and preprocesses ECG data, segments it into beats, and extracts features.

- Steps:

  - Reads the ECG file.

  - Removes mean and applies filtering.

  - Detects QRS complexes and segments the signal into beats.

  - Extracts wavelet-based features.

- Why?: Prepares raw ECG signals for model training and testing by transforming them into a feature matrix.

## 4. Classification

### 4.1 KNeighborsClassifier

- Purpose: Classifies ECG signals using the k-nearest neighbors (KNN) algorithm.

- Parameters:

  - n_neighbors: Number of neighbors to consider (default: 2).

  - metric: Distance metric, such as Euclidean.

- Why?: KNN effectively distinguishes between normal and LBBB signals when the feature space boundaries are well-separated.

## 5. Prediction

### 5.1 predict_ecg(file_path, Fs=1000)

- Purpose: Predicts whether an input ECG file contains normal beats or LBBB.

- Steps:

  - Preprocesses the signal.

  - Segments it into beats.

  - Extracts features using wavelets.

  - Predicts labels for each beat using KNN.

  - Computes the majority vote for final classification.

- Why?: Provides an automated pipeline for real-time classification of ECG signals.

## Why These Steps for LBBB Detection?

### Preprocessing

- LBBB manifests as prolonged QRS complexes. Preprocessing ensures these features are preserved by removing noise and baseline drift.

### Wavelet Features

- Wavelet transforms capture transient features like the QRS complex. Daubechies wavelets (e.g., 'db3') are particularly effective for this task.

### KNN Classifier

- The simplicity and effectiveness of KNN make it suitable for small datasets with clear class boundaries, such as LBBB vs. normal beats.

## Suggestions for Enhancements

### Additional Models

- Support Vector Machines (SVM): Offers robust classification with proper kernel selection.

- Random Forests: Handles non-linear feature spaces effectively.

- Deep Learning: End-to-end models like CNNs can automatically learn features from raw ECG signals.

### Additional Features

- Incorporate heart rate variability (HRV) metrics for enhanced classification.

- Use higher-order wavelet coefficients to improve discriminatory power.

### Improved Preprocessing

- Apply adaptive filtering for dynamic noise removal.

- Segment beats dynamically based on R-R intervals for better alignment.

## Conclusion

This pipeline combines signal preprocessing, wavelet feature extraction, and a simple yet effective KNN classifier to classify ECG signals into normal and LBBB categories. It provides a robust foundation for detecting LBBB and can be further enhanced with advanced models and features.